- 混频器和乘法器

Analog Devices, Inc.（ 简称ADI）的混频器和乘法器具有高性能指标（例如速度和动态范围）以及较高集成度的功能性。这些赋予无线电设计新型和非常理想的体系结构改变，从而减少外围元器件数量，降低成本并且易于设计。

Loading Selection Table Data. Please wait...

Processing request. Please wait.

- Help

- Sort: Click on the arrows beneath the parameter on which you would like to sort, ascending and descending order.

- Add/Remove Parameters: Click on the Add/Remove Parameters button. Select/deselect the checkbox of the parameter in the add/remove parameter pop up box. Click on "Update" for the table to redraw with new selection of parameters.

- Filter: Enter the desired value to filter parameter(s) by, and click the "Update Results" button to update the table display.

- Reset Table: Click "Reset Table" to clear all filters/sorts and restore table back to default view.

- Quick Filters: When applicable, select a quick filter with preset parameters filtered to narrow the results returned.

- Mixers/Multipliers

hide

- Add/Remove Parameters

RF Frequency (MHZ) 
IF Frequency (MHz) 
LO Frequency (MHz)

LO Drive (dBm) 
Conversion Gain (dB) 
IP3 (dBm)

P1dB (dBm) 
NF (dB)

Pos Supply 
Supply Current (max) 
Package

Price* (1000-4999)

| Part# R: 7 | RF Frequency (MHZ) | IF Frequency (MHz) | LO Frequency (MHz) | LO Drive (dBm) | IP3 (dBm) | P1dB (dBm) | NF (dB) | Pos Supply | Supply Current (max) | Package | Price* (1000-4999) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AD831 | 400 | 200 | 400 | 0 | 24 | 10 | 14 | +9V to +11V | 100 | 20-Lead PLCC | $6.83 |
| AD8342 | LF to 2400 | LF to 2400 | LF to 2400 | 0 | 24 | 8.5 | 12 | +4.75V to +5.25V | 97 | 16-Lead LFCSP | $3.59 |
| AD8343 | dc to 2500 | dc to 2500 | dc to 2500 | -10 | 16.5 | 2.8 | 14 | +4.5V to +5.5V | 50 | 14-Lead TSSOP | $3.67 |
| AD8344 | 400 to 1200 | 70 to 400 | 470 to 1600 | 0 | 24 | 8.5 | 10 | +4.75V to +5.25V | 84 | 16-Lead LFCSP | $3.59 |
| ADL5350 | LF to 4000 | LF to 4000 | LF to 4000 | +4 | 25 | 19 | 6.5 | +2.7V to +3.5V | 19 | 8-Lead LFCSP | $2.39 |
| ADL5390 | 20 to 2400 | 230 | 20 to 2400 | - | 25 | 13 | 21 | +4.75V to +5.25V | 130 | 24-Lead LFCSP | $7.59 |
| ADL5391 | dc to 2000 | - | - | - | 14 | 13.2 | - | +4.5V to +5.5V | 135 | 16-Lead LFCSP | $4.95 |